## IP Configuration and VLAN Implementation for 
ITSKIILCENTER Enterprise Network

## Network Design and Topology

A small enterprise network was designed for ITSKIILCENTER using Cisco Packet Tracer. The network consisted of one Cisco 2901 router, one Cisco switch, and six computers representing employees from three different departments: Administration, Training, and IT Support.

The router was connected to the switch using a Gigabit Ethernet connection, while all computers were connected to the switch through Fast Ethernet ports. The purpose of this design was to simulate a real-world business environment where different departments require network separation while still being able to communicate when necessary.

## VLAN Implementation

To improve network organization and security, Virtual Local Area Networks (VLANs) were created. Each department was assigned its own VLAN and network segment.

| VLAN ID | VLAN Name | Department |
| --- | --- | --- |
| 10 | ADMIN | Administration |
| 20 | TRAINING | Training |
| 30 | IT_SUPPORT | IT Support |

The VLANs were created on the switch and the corresponding switch ports were assigned to each department. Ports Fa0/1 and Fa0/2 were assigned to the Administration VLAN, Fa0/3 and Fa0/4 to the Training VLAN, and Fa0/5 and Fa0/6 to the IT Support VLAN.

This segmentation ensures that broadcast traffic remains within each department and improves both network performance and security.

## IP Addressing Scheme

An IP addressing plan was developed before configuration began. Each VLAN was assigned its own subnet and default gateway.

| VLAN | Network Address | Subnet Mask | CIDR | Gateway |
| --- | --- | --- | --- | --- |
| VLAN 10 (ADMIN) | 192.168.10.0 | 255.255.255.0 | /24 | 192.168.10.1 |
| VLAN 20 (TRAINING) | 192.168.20.0 | 255.255.255.0 | /24 | 192.168.20.1 |
| VLAN 30 (IT_SUPPORT) | 192.168.30.0 | 255.255.255.0 | /24 | 192.168.30.1 |

A /24 subnet mask (255.255.255.0) was selected because each department required only a small number of devices while still allowing for future growth. A /24 network provides up to 254 usable host addresses, making it suitable for the ITSKIILCENTER environment.

| Device | IP Address | Default Gateway |
| --- | --- | --- |
| ADMIN-PC1 | 192.168.10.10 | 192.168.10.1 |
| ADMIN-PC2 | 192.168.10.11 | 192.168.10.1 |
| TRAIN-PC1 | 192.168.20.10 | 192.168.20.1 |
| TRAIN-PC2 | 192.168.20.11 | 192.168.20.1 |
| IT-PC1 | 192.168.30.10 | 192.168.30.1 |
| IT-PC2 | 192.168.30.11 | 192.168.30.1 |

Each VLAN required its own gateway because every VLAN represents a separate network. The gateways were later configured on the router using sub-interfaces.

## Trunk Configuration

After VLAN creation, the connection between the router and switch was configured as a trunk link. A trunk port allows traffic from multiple VLANs to travel through a single physical cable.

Initially, communication between VLANs failed because the switch port connected to the router was mistakenly configured as an access port. After troubleshooting, the configuration was changed to trunk mode, which allowed VLAN traffic to be forwarded correctly to the router.

This was the main issue encountered during implementation and provided valuable experience in network troubleshooting.

## Inter-VLAN Routing

To enable communication between departments, Router-on-a-Stick technology was implemented. Instead of using separate physical interfaces for each VLAN, a single router interface was divided into multiple logical sub-interfaces.

Each sub-interface was assigned a VLAN tag using the IEEE 802.1Q protocol and configured with an IP address to act as the default gateway for its respective VLAN.

The router therefore served as:

- Gateway for VLAN 10: 192.168.10.1

- Gateway for VLAN 20: 192.168.20.1

- Gateway for VLAN 30: 192.168.30.1

This configuration allowed devices from different VLANs to communicate through the router while maintaining network segmentation.

## DNS Configuration

To simulate a realistic enterprise network environment, a DNS server address was configured on all end devices. Google Public DNS (8.8.8.8) was used because it is a reliable and widely available public DNS service.

The DNS configuration allows client devices to resolve domain names into IP addresses and represents an important component of a complete network configuration.

| Device Group | DNS Server |
| --- | --- |
| Administration VLAN | 8.8.8.8 |
| Training VLAN | 8.8.8.8 |
| IT Support VLAN | 8.8.8.8 |

## Connectivity Testing

After completing the configuration, several connectivity tests were performed using the ping command.

Testing confirmed that:

- Devices within the same VLAN could communicate successfully.

- Devices could reach their assigned default gateway.

- Devices in different VLANs could communicate through the router.

Successful ping results verified that VLAN segmentation, trunking, IP addressing, and inter-VLAN routing had been configured correctly.

## Challenges Encountered

During the implementation process, inter-VLAN communication initially failed. After troubleshooting, it was discovered that the switch port connected to the router had been configured as an access port rather than a trunk port. Since a trunk port is required to carry traffic from multiple VLANs, the configuration was corrected using the switchport mode trunk command. Once the issue was resolved, successful communication between VLANs was established.

## Skills Learned

Through this project, I gained practical experience in:

- VLAN creation and management

- IP addressing and subnetting

- Switch port configuration

- Trunking concepts

- Router-on-a-Stick implementation

- Inter-VLAN routing

- Network troubleshooting and connectivity testing

## Conclusion

The ITSKIILCENTER enterprise network was successfully designed and implemented using VLAN technology and Router-on-a-Stick routing. Three separate departments were isolated into their own VLANs while still maintaining controlled communication through inter-VLAN routing. The project demonstrated key networking concepts including VLAN segmentation, IP addressing, trunk configuration, default gateways, and troubleshooting techniques. Through testing and validation, the network met all project requirements and functioned as intended.